Discovery 10: Implement OSPFv3

Introduction

Finally, you will then optimize OSPFv3 functionality by:

- Advertising a default route into the OSPFv3 domain

- Tuning OSPFv3 interface costs

- Configuring passive interfaces

- Advertising IPv6 summary routes

R1 and R4 are connected to a common multiaccess GigabitEthernet segment. R1 and R2 are connected over a point to point serial link. R1 and R3 are connected over an GigabitEthernet WAN link. The discovery is prepared with the devices that are represented in the topology diagram and configured according to the information in the Job Aids section. All devices have their basic configurations in place, including hostnames and IP addresses.

Your configuration tasks are as follows:

- Enable traditional OSPFv3 on R1

- Migrate to OSPFv3 Address Families on R1

- Optimize OSPFv3 behavior

Task 1: Enable Traditional OSPFv3 on R1

Activity

Note

All addressing and naming conventions used in these Learning Points will differ from the actual lab steps that you will perform.

Enable IPv6 routing.

Router(config)# ipv6 unicast-routing

Enable the OSPFv3 routing process.

Router(config)# ipv6 router ospf process-id

Configure the router ID.

Router(config-router)# router-id router-id

Enable OSPFv3 on an interface.

Router(config-if)# ipv6 ospf process-id area area-id

(Optionally): Configure passive interfaces by using the default command or configure passive interfaces by using the interface command.

Router(config-router)# passive-interface default Router(config-router)# no passive-interface interface slot/number

OR

Router(config-router)# passive-interface interface slot/number

You need to take the following steps to configure traditional OSPFv3 on a router:

Because IPv6 routing is not enabled by default, you must first enable it using the ipv6 unicast-routing command.

Next you must enable the OSPFv3 routing process with the selected process-id parameter.

If there are no IPv4 addresses configured on the router, the OSPFv3 routing process requires that you manually configure the router ID.

Optionally, you can configure passive interfaces.

All that remains then is for you to enable OSPFv3 routing on the interface. Of course, the interface must have an IPv6 address assigned and it has to be administratively enabled.

The table defines the commands that you use to configure traditional OSPFv3.

| OSPFv3 Configuration Commands |  |
| --- | --- |
| Command | Description |
| ipv6 unicast-routing | Enables the forwarding of IPv6 unicast datagrams and is used in the global configuration mode. To disable the forwarding of IPv6 unicast datagrams, use the no form of this command. |
| ipv6 router ospf process-id | Enables OSPF for the IPv6 router configuration mode. The process-id value is an internal identification. It is locally assigned and can be a positive integer from 1 to 65,535. |
| router-id router-id | Executed in the OSPF router configuration mode to statically configure a router ID, which is the name for the router within the OSPFv3 process. |
| ipv6 ospf process-id area area-id | Enables OSPFv3 on an interface and assigns it to the specified area. |
| passive-interface default | Configures all interfaces as passive for the OSPFv3 process. |
| passive-interface interface slot/number | Configures a specified interface as passive for the OSPFv3 process. |

Step 1: On R1, enable IPv6 unicast routing and start an IPv6 OSPF router process with ID 1. Configure a router ID of 1.1.1.1. Declare Loopback 0 as a passive interface.

R1(config)# ipv6 unicast-routing

R1(config)# ipv6 router ospf 1

R1(config-rtr)# router-id 1.1.1.1

R1(config-rtr)# passive-interface Loopback0

OSPFv3 is the IPv6-capable version of the OSPF routing protocol. It is a rewrite of the OSPF protocol to support IPv6, although the foundation remains the same as in IPv4 and OSPFv2. The OSPFv3 metric is still based on interface cost. The packet types and neighbor discovery mechanisms are the same in OSPFv3 as they are for OSPFv2. OSPFv3 also supports the same interface types, including broadcast, point-to-point, point-to-multipoint, nonbroadcast multiaccess, and virtual links. LSAs are still flooded throughout an OSPF domain, and many of the LSA types are the same, though a few have been renamed or newly created.

Cisco IOS routers offer two OSPF configuration methods for IPv6: using the traditional ipv6 router ospf approach or the new router ospfv3 process. You will first examine the traditional configuration approach and then migrate the configuration to the new style.

To start any IPv6 routing protocols, you need to enable IPv6 unicast routing using the ipv6 unicast-routing command. In the traditional configuration approach, the OSPFv3 and OSPFv2 processes run independently on a router. In the traditional way, you start the OSPF process for IPv6 using the ipv6 router ospf command.

The OSPF process for IPv6 no longer requires an IPv4 address for the router ID, but it does require a 32-bit number to be set. You define the router ID using the router-id command. If you do not set the router ID, the system will try to dynamically choose an ID from the currently active IPv4 addresses. If there is no active IPv4 address, the process will fail to start.

In the IPv6 router ospf configuration mode you can specify the passive interfaces (using the passive-interface command), enable summarization, and fine-tune the operation, but you do not enable the process on specific interfaces. There is no network command. To activate the OSPF process on required interfaces, you will need to use the ipv6 ospf command in interface configuration mode.

Step 2: On R1, enable the OSPF-for-IPv6 process on all active interfaces except for Loopback 1. Assign interface Loopback 0 and GigabitEthernet 0/0 to Area 0, GigabitEthernet 0/2 to Area 1, and GigabitEthernet 0/1 to area 2. Apply the OSPF point-to-point network type to the Loopback 0 interface.

R1(config)# interface Loopback0

R1(config-if)# ipv6 ospf 1 area 0

R1(config-if)# ipv6 ospf network point-to-point

R1(config-if)# interface G0/0

R1(config-if)# ipv6 ospf 1 area 0

R1(config-if)# interface G0/2

R1(config-if)# ipv6 ospf 1 area 1

R1(config-if)# interface G0/1

R1(config-if)# ipv6 ospf 1 area 2

IPv6 address differs from IPv4 addresses. You have multiple IPv6 interfaces on a single interface: a link-local address, one or more global addresses and others. OSPF communication within a local segment is based on link-local addresses, and not global addresses. These differences are one of the reasons why you enable the OSPF process per interface in the interface configuration mode and not with the network command.

To enable the OSPF-for-IPv6 process on an interface and assign that interface to an area, use the ipv6 ospf ospf-process area area-id command in the interface configuration mode. To be able to enable OSPFv3 on an interface, the interface must be enabled for IPv6. This implementation is typically achieved by configuring a unicast IPv6 address. Alternatively, you could also enable IPv6 using the ipv6 enable interface command, which will cause the router to derive its link-local address.

By default, OSPF for IPv6 will advertise a /128 prefix length for any Loopback interfaces that are advertised into the OSPF domain. The ipv6 ospf network point-to-point command ensures that the 2001:DB8:0:11::/64 prefix is advertised with the correct prefix length (64 bits) instead of a prefix length of 128.

Notice the three OSPF neighbor adjacency change messages that are displayed at the console when you enable the process on the physical interface connected to other routers.

Step 3: On R1, examine the OSPFv3 adjacencies and routing table.

R1# show ipv6 ospf neighbor

R1# show ipv6 route ospf

After enabling the OSPF process on IPv6 interfaces, you can verify the adjacencies and the IPv6 routing table. You can selectively display the OSPF-populated part of the routing table, if you use the show ipv6 route command with the ospf keyword. From R1’s perspective, all routes appear is intra-area routes in its routing table since it is connected to all OSPF areas in the routing domain (GigabitEthernet 0/0 = Area 0; GigabitEthernet 0/2 = Area 1; GigabitEthernet 0/1 = Area 2).

All router interfaces are manually configured with predictable link-local addresses:

- R1 = FE80::1

- R2 = FE80::2

- R3 = FE80::3

- R4 = FE80::4

Step 4: On R1, examine the IPv6 OSPF database.

R1# show ipv6 ospf database

OSPFv3 (for IPv6) renames two LSA types and defines two additional LSA types that do not exist in OSPFv2 (for IPv4).

The two renamed LSA types are:

- Interarea prefix LSAs for ABRs (Type 3): Type 3 LSAs advertise internal networks to routers in other areas (interarea routes). Type 3 LSAs may represent a single network or a set of networks summarized into one advertisement. Only ABRs generate summary LSAs. In OSPF for IPv6, addresses for these LSAs are expressed as prefix/prefix length instead of address, and mask. The default route is expressed as a prefix with length 0.

- Interarea router LSAs for ASBRs (Type 4): Type 4 LSAs advertise the location of an ASBR. An ABR originates an Inter-Area Router LSA into an area to advertise an ASBR that resides outside of the area. The ABR originates a separate Inter-Area Router LSA for each ASBR it advertises. Routers that are trying to reach an external network use these advertisements to determine the best path to the next hop.

The two new LSA types are:

- Link LSAs (Type 8): Type 8 LSAs have local-link flooding scope and are never flooded beyond the link with which they are associated. Link LSAs provide the link-local address of the router to all other routers that are attached to the link. They inform other routers that are attached to the link of a list of IPv6 prefixes to associate with the link. In addition, they allow the router to assert a collection of option bits to associate with the network LSA that will be originated for the link.

- Intra-area prefix LSAs (Type 9): A router can originate multiple intra-area prefix LSAs for each router or transit network, each with a unique link-state ID. The link-state ID for each intra-area prefix LSA describes its association to either the router LSA or the network LSA. The link-state ID also contains prefixes for stub and transit networks.

Task 2: Migrate to OSPFv3 Address Families on R1

Activity

OSPFv3 does not only support exchange of IPv6 routes, it also supports exchange of IPv4 routes.

- Single process can support IPv4 and IPv6

- Single database with LSAs carries IPv4 and IPv6 information

- Adjacencies are established individually for each address family

- Specific settings are configured within address families (IPv4/IPv6)

- The OSPFv3 address families feature is supported as of Cisco IOS Release 15.1(3)S and Cisco IOS Release 15.2(1)T

The newest OSPFv3 configuration approach utilizes a single OSPFv3 process. It is capable of supporting IPv4 and IPv6 within that single OSPFv3 process. OSPFv3 builds a single database with LSAs that carry IPv4 and IPv6 information. The OSPF adjacencies are established separately for each address family. Settings that are specific to an address family (IPv4/IPv6) are configured inside that address family router configuration mode.

Running a single OSPFv3 for both IPv4 and IPv6 is supported in Cisco IOS since software release 15.1(3)S.

Step 1: On R1, configure an OSPFv3 process using address family configuration style (router ospfv3). Use process number 1 with an OSPF router ID 1.1.1.1, and make the Loopback 0 interface passive.

R1(config)# router ospfv3 1

R1(config-router)# router-id 1.1.1.1

R1(config-router)# passive-interface Loopback0

You can enable the new OSPFv3 process using the router ospfv3 process-number command. Within the OSPF process configuration mode, you can set the OSPF process ID (using the router-id command), set the passive interfaces, and tune the per-process OSPF behavior.

Step 2: Examine the OSPFv3 router configuration.

R1# show running-config | section router

You can view the resulting OSPF process configuration using the show running-config | section router command. The old OSPF router configuration (ipv6 router ospf) has disappeared and has been replaced by the new router ospfv3 with an address-family submode.

The router ID is displayed in the router configuration mode making it valid globally for all address families. It also appears under the IPv6 address family. It is possible to have different router IDs for different address families.

The address-family ipv6 unicast address family has been automatically created on R1. Cisco IOS Software has parsed the previous OSPFv3 configuration and found that the OSPF process was enabled only for IPv6. Therefore, when you enable the new configuration method, the IPv6 address family has been instantiated and the IPv4 address family does not show in the configuration.

The passive-interface configuration is a setting that is valid per address family. You can have dissimilar settings for IPv4 and IPv6. Therefore, this command has been placed in the address-family submode.

Step 3: Examine OSPFv3 operation by verifying the adjacencies, routing table, and database.

R1# show ospfv3 neighbor

R1#show ipv6 route ospf

R1# show ospfv3 database

You can verify the OSPFv3 operation using the old commands (show ipv6 ospf neighbor, show ipv6 ospf database) or the new style of commands, such as show ospfv3 neighbor and show ospfv3 database.

Despite the change of the OSPFv3 configuration to the new approach, the OSPF connectivity has been retained. In fact, R1 now uses a mixed configuration: new style of process configuration, and the old style of interface commands. The old interface commands are:

interface Loopback0 ipv6 ospf 1 area 0 ipv6 ospf network point-to-point ! ipv6 ospf 1 area 0 ! interface GigabitEthernet0/1 ipv6 ospf 1 area 2 ! interface G0/2 ipv6 ospf 1 area 1

Step 4: On R1, enable the OSPFv3 IPv6 address family on all active interfaces except Loopback 1 using the new configuration approach.

R1(config)# interface Loopback 0

R1(config-if)# ospfv3 1 ipv6 area 0

R1(config-if)# ospfv3 1 ipv6 network point-to-point

R1(config-if)# interface G 0/0

R1(config-if)# ospfv3 1 ipv6 area 0

R1(config-if)# interface G0/2

R1(config-if)# ospfv3 1 ipv6 area 1

R1(config-if)# interface G 0/1

R1(config-if)# ospfv3 1 ipv6 area 2

The preferred interface mode command for the new style OSPFv3 configuration is the ospfv3 process-id {ipv4|ipv6} area area-id command. It allows you to selectively activate the OSPFv3 process for an address family (IPv4 or IPv6) on a given interface.

With the OSPFv3 address families feature, you may have two device processes per interface, but only one process per address family. If the IPv4 address family is used, an IPv4 address must first be configured on the interface. For the IPv6 address family, it is enough, if only IPv6 is enabled on the interface, as OSPFv3 uses link-local addresses. A single IPv4 or IPv6 OSPFv3 process running multiple instances on the same interface is not supported.

The OSPF point-to-point network type is configured for the IPv6 address family on the Loopback 0 interface.

Step 5: On R1, verify the resulting configuration and operation.

R1# show running-config interface Loopback 0 | include ospfv3

ospfv3 1 ipv6 area 0

ospfv3 1 ipv6 network point-to-point

R1# show running-config interface GigabitEthernet 0/0 | include ospfv3

ospfv3 1 ipv6 area 0

R1# show running-config interface G 0/2 | include ospfv3

ospfv3 1 ipv6 area 1

R1# show running-config interface GigabitEthernet 0/1 | include ospfv3

ospfv3 1 ipv6 area 2

You can view the configuration of an interface using the show running-config interface command. If you want to display only the interface commands that include a certain keyword, you can filter the command output using the include keyword.

The configuration on R1 shows that the new-style interface-mode commands have replaced the old-style (ipv6 ospf) commands.

The OSPF operation has not been affected. The OSPFv3 adjacencies, database, and routing table are functional.

Step 6: On R1, enable the running OSPFv3 process for IPv4. Make Loopback 0 a passive interface.

R1(config)# router ospfv3 1

R1(config-router)# address-family ipv4 unicast

R1(config-router-af)# passive-interface Loopback0

R1(config-router-af)# interface Loopback0

R1(config-if)# ospfv3 1 ipv4 area 0

R1(config-if)# ospfv3 1 ipv4 network point-to-point

R1(config-if)# interface GigabitEthernet0/0

R1(config-if)# ospfv3 1 ipv4 area 0

R1(config-if)# interface GigabitEthernet0/1

R1(config-if)# ospfv3 1 ipv4 area 2

R1(config-if)# interface G0/2

R1(config-if)# ospfv3 1 ipv4 area 1

To activate OSPFv3 for IPv4, you need to configure the ospfv3 process-number ipv4 area area-id command in the configuration mode of the desired interface.

This way some (or all) of the links may be allowed to do IPv4 forwarding and be configured with IPv4 addresses. For example, pockets of IPv4-only devices may exist around the edges running an IPv4 static or dynamic routing protocol. In that scenario, you could forward IPv4 or IPv6 traffic between these pockets. The transit device needs both IPv4 and IPv6 forwarding stacks (that is, a dual stack).

This feature allows a separate (possibly incongruent) topology to be constructed for the IPv4 address family. It installs IPv4 routes in the IPv4 RIB, and then the forwarding occurs natively. The OSPFv3 process fully supports an IPv4 AF topology and can redistribute routes from and into any other IPv4 routing protocol.

An OSPFv3 process can be configured to be IPv4 or IPv6. The address-family command is used to determine which AF will run in the OSPFv3 process. Once the address family is selected, you can enable multiple instances on a link and enable address-family-specific commands.

Loopback 0 is configured as passive under the IPv4 address family, as it was for the IPv6 address family. The Loopback 0 interface is also configured with the OSPF point-to-point network type to ensure that OSPF advertises the correct /24 subnet mask instead of /32.

Step 7: On R1, examine the OSPFv3 adjacencies.

R1# show ospfv3 neighbor

You can monitor the IPv4 routing table that is computed from the OSPFv3 database using the show ip route ospfv3 command. The ospfv3 keyword filters the content of the routing table and displays only the OSPFv3 routes.

Note that the command show ip route ospf will not show any routes since that command references the OSPFv2 protocol.

Step 8: On R1, examine the IPv4 routing table that is computed from the OSPFv3 database.

R1#show ip route ospfv3

You can monitor the IPv4 routing table that is computed from the OSPFv3 database using the show ip route ospfv3 command. The ospfv3 keyword filters the content of the routing table and displays only the OSPFv3 routes.

Note that the command show ip route ospf will not show any routes since that command references the OSPFv2 protocol.

Step 9: On R1, examine the OSPFv3 database.

R1# show ospfv3 database inter-area prefix

A router maintains a single OSPFv3 database, which contains various LSAs. Some LSAs carry IPv4-related information, others carry IPv6-related information, and others carry mixed information. You must examine specific LSA types to see which address family is described by a given LSA.

Note that the old style of verification command like show ip ospf database will not show any information.

Task 3: Optimize OSPFv3 Behavior

Activity

OSPFv3 offers you a set of tools that is very similar to that of OSPFv2. These tools allow you to fine-tune the OSPFv3 functionality.

IPv6 route summarization on ABR

R1(config)# router ospfv3 1

R1(config-router)# address-family ipv6 unicast

R1(config-router-af)# area 1 range 2001:DB8:ACAD::/60

IPv6 route summarization on ASBR

R1(config)# router ospfv3 1

R1(config-router)# address-family ipv6 unicast

R1(config-router-af)# summary-prefix 2001:DB8:1::/56

IPv6 default route advertisement

R1(config)# router ospfv3 1

R1(config-router)# address-family ipv6 unicast

R1(config-router-af)# default-information originate

Step 1: On R3 and R4, view the IPv6 routing tables. What approach can you take to reduce their size?

R3# show ipv6 route ospf

R4# show ipv6 route ospf

When you display the IPv6 routing tables on R3 and R4, you will see numerous OSPF interarea routes. You could reduce the size of the R3 and R4 routing table by summarizing the inter-area routes on the Area Border Router.

Step 2: Summarize the IPv6 networks advertised by R2 (2001:DB8:0:220::/64 to 2001:DB8:0:22F::/64) using the smallest possible address block.

R1(config)# router ospfv3 1

R1(config-router)# address-family ipv6 unicast

R1(config-router-af)# area 1 range 2001:DB8:0:220::/60

Like in IPv4, OSPFv3 supports IPv6 address summarization. You can summarize interarea routes on the Area Border Routers using the area area-id range command in the desired address family mode. In this scenario, you summarize a set of IPv6 network addresses using the address block 2001:DB8:0:220::/60.

Although not demonstrated in this discovery, you can summarize external routes on the ASBRs. To perform such summarization for IPv6, you would use the summary-prefix command in the IPv6 address family router configuration mode.

Step 3: Verify the IPv6 summarization effects on R3 and R4.

R3# show ipv6 route ospf

R4# show ipv6 route ospf

You can verify the summarization effects by viewing the IPv6 routing table on the R3 and R4 routers. Notice the summary address 2001:DB8:0:220::/60 instead of the individual smaller networks.

Step 4: On R4, verify the OSPFv3 routing table details relating to the 2001:DB8:0:22::/64 prefix.

R4# show ipv6 route 2001:DB8:0:22::/64

The total cumulative cost for R4 to reach the 2001:DB8:0:22::/64 summary is 75. This represents a cost of 10 for the GigabitEthernet link between R4 and R1, plus a cost of 64 for the serial link between R1 and R2, plus a cost of 1 for the Loopback interface on R2 (10+64+1=75).

Step 5: On R1, increase the OSPF cost of the serial interface link to 200.

R1(config)# interface G 0/2

R1(config-if)# ospfv3 1 ipv6 cost 200

The new cost value is applied specifically to the OSPFv3 IPv6 address family only. The interface cost under the IPv4 address family will remain 64.

Step 6: On R4, verify the OSPFv3 routing table to see the new cost assigned to the 2001:DB8:0:22::/64 prefix.

R4# show ipv6 route 2001:DB8:0:22::/64

Routing entry for 2001:DB8:0:22::/64 Known via "ospf 1", distance 110, metric 211, type inter area Route count is 1/1, share count 0 Routing paths: FE80::1, GigabitEthernet0/0 Last updated 02:07:09 ago

The new calculated metric is now 211 (10 + 200 + 1).

Step 7: On R1, advertise a default route into the OSPFv3 IPv4 and IPv6 address families.

R1(config)# router ospfv3 1

R1(config-router)# address-family ipv4 unicast

R1(config-router-af)# default-information originate

R1(config-router-af)# address-family ipv6 unicast

R1(config-router-af)# default-information originate

R1 is already configured with default static routes pointing to its Loopback 1 interface. To generate a default external route into an OSPF routing domain, use the default-information originate router configuration command under the appropriate address family.

Step 8: On R3, verify the IPv4 and IPv6 routing tables to ensure that the default route is successfully being advertised into the OSPF domain. Ping R1’s Loopback 1 interface to very reachability.

R3# show ip route ospfv3

R3# show ipv6 route ospf

R3#ping 209.165.201.1

R3#ping 2001:DB8:209:165::1

The default route is being advertised throughout the OSPF domain for both IPv4 and IPv6 address families. Pings to the R1 Loopback 1 IPv4 and IPv6 addresses are successful.

Step 9: On R4, investigate the hello and dead timer values on GigabitEthernet 0/0 using the show ospfv3 interface command.

R4# show ospfv3 interface GigabitEthernet 0/0

Notice that the OSPFv3 interface details appear twice – once for the IPv4 address family and once for the IPv6 address family (in that order). The hello and dead times are identical for both address families: 10 seconds for the hello timer, and 40 seconds for the dead timer. It is possible to adjust the hello and dead timer values using the ospfv3 process-id {ipv4 | ipv6} hello-interval interval-value or ospfv3 process-id {ipv4 | ipv6} dead-interval interval-value commands.