Discovery 9: Apply OSPF Optimization

Overview

Through this discovery, you will explore OSPF optimization by first implementing route summarization on ABRs and ASBRs in an OSPF environment, and then examining the impact of summarization on the OSPF database and internal routing.

You will also explore different methods of applying route filtering to OSPF, on both ABRs and ASBRs.

The discovery is prepared with the devices that are represented in the topology diagram and configured according to the information in the Job Aids section. You have access to all four routers. They have a basic configuration in place, including hostnames and IP addresses. OSPF has been configured on all routers. Router R3 is advertising four external networks into OSPF. Router R4 is advertising a default route into OSPF.

Your configuration tasks are as follows:

- Configure OSPF ABR route summarization

- Configure OSPF ASBR route summarization

- Configure OSPF Filtering

Task 1: Configure OSPF ABR Route Summarization

OSPF ABR Route Summarization

Summarization of type 3 summary LSAs means that we are creating a summary of all the interarea (type 1 and type 2 LSAs) routes. This is why you call it interarea route summarization.

To configure route summarization on ABR, use the following commands:

router( config-router) # area area-id range address-mask [ advertise | not-advertise] [cost cost]

A summary route will only be advertised if you have at least one prefix that falls within the summary range. The cost of summary route will have the cost of the prefix with the lowest cost that falls within the summary range. Also, the ABR that creates the summary route will create a Null0 interface to prevent loops.

Step 1: On R1, examine the OSPF routing tables.

On R1, enter the following command:

R1# show ip route ospf

Apart from the loopback networks (192.168.x.0/24, where x is the router ID), you will see eight Class C networks that are advertised by R2 as intra-area routes (192.168.16.0/24 to 192.168.23.0/24) and four class A networks that are advertised by R3 as O E2 external routes (10.33.4.0/24 to 10.33.7.0/24). You will also notice that R1 is receiving a 0.0.0.0/0 default route from R4.

Step 2: On R4, investigate the OSPF routing table.

On R4, enter the following command:

R4# show ip route ospf

You can see the same internal networks that are listed as O IA interarea summary routes. They are being flooded into each area without any summarization on the area borders. You can also see the external networks that are still listed as O E2 external routes since Type-5 AS External LSAs are flooded throughout the OSPF domain by default.

Note

You can see the respective routes that are received from the other areas on R2 and R3 as well.

Step 3: On R4, examine the OSPF database.

On R4, enter the following command:

R4# show ip ospf database

You will find corresponding Type-3 LSAs (Summary Net Link States) and Type-5 AS External LSAs for each interarea summary and external networks that are advertised from R1.

Step 4: On R1, summarize the eight internal networks in Area 1 (192.168.16.0/24 to 192.168.23.0/24) using the appropriate address block.

On R1, enter the following commands:

R1(config)# router ospf 1

R1(config-router)# area 1 range 192.168.16.0 255.255.248.0

OSPF is a classless routing protocol, which carries subnet mask information along with route information. Therefore, OSPF supports multiple subnet masks for the same major network, which is known as VLSM. OSPF also supports discontiguous subnets, because the subnet masks are part of the LSDB. Network numbers in areas should be assigned contiguously to ensure that these addresses can be summarized into a minimal number of summary addresses.

In this scenario, the list of eight networks that are advertised by R2 (192.168.16.0/24 to 192.168.23.0/24) in the routing table of the ABR can be summarized into one address block. All these networks will be summarized on the ABR R1. The block of addresses from 192.168.16.0 through 192.168.23.0/24 can be summarized using 192.168.16.0/21.

To consolidate and summarize routes at an area boundary, use the area range command in OSPF router configuration mode. The ABR will summarize routes for a specific area before injecting them into a different area via the backbone as Type-3 summary LSAs.

Step 5: Examine the OSPF routing tables on R3 and R4 with Area 1 route summarization enabled on R1.

On R3 and R4, enter the following commands:

R3# show ip route ospf

R4# show ip route ospf

Apart from the loopback networks and the external networks, you will see the summary block of Area 1 networks.

Step 6: Verify the OSPF database on the backbone router R4.

On R4, enter the following command:

R4# show ip ospf database

You will see the Type-3 LSA for the summarized address block from Area 1 being advertised into Area 0. The Type-3 LSAs for the individual networks are no longer in the database.

Step 7: Examine the OSPF routing table on R1 for a route with the Null0 interface.

On R1, enter the following command:

R1# show ip route ospf

To prevent routing loops, Cisco IOS Software creates a summary route to the Null0 interface when manual summarization is configured. For example, if the summarizing router receives a packet to an unknown subnet that is part of the summarized range, the packet matches the summary route that is based on the longest match. The packet is forwarded to the Null0 interface (in other words, it is dropped), which prevents the router from forwarding the packet to a default route and possibly creating a routing loop.

Task 2: Configure OSPF ASBR Route Summarization

Activity

Summarization on ASBR

As you have discovered in the previous task, R3 is redistributing external networks and advertising them to R1 using Type-5 AS External LSAs. R1 floods this information across the backbone and into other regular OSPF areas. Each individual prefix is carried in its own LSA.

It is possible to summarize these external networks. This would minimize the number of routing table entries, reduce LSA flooding, and save CPU resources. It would also localize the impact of any topology changes if an external network fails. In this example, R3 is the ASBR redistributing the external networks. Since it is responsible for originating the individual Type-5 AS External LSAs for these networks, you can configure summarization on R3.

To configure route summarization on ASBR, use the following commands:

router( config-router) # summary address ip address mask [advertise |not-advertise]

Step 1: On R3, summarize the four IPv4 external networks using the smallest possible address block that covers all individual subnets.

On R3, enter the following commands:

R3(config)# router ospf 1

R3(config-router)# summary-address 10.33.4.0 255.255.252.0

The block of addresses from 10.33.4.0/24 through 10.33.7.0/24 can be summarized using 10.33.4.0/22.

To consolidate and summarize routes at an ASBR, use the summary-address command. The ASBR will summarize external routes before injecting them into area 2 as a single Type-5 AS External LSA.

Step 2: On R3, examine the OSPF routing tables.

On R3, enter the following command:

R3# show ip route ospf

Notice that the routing table on R3 now carries an OSPF entry for the summary and that it points to the Null0 interface. This is the same loop prevention mechanism described in Task 1.

Step 3: On R1, R2, and R4, examine the OSPF routing tables, and verify the OSPF database on R4. Verify connectivity to the external networks.

On R1, R2, and R4, enter the following commands:

R1# show ip route ospf

R2# show ip route ospf

R4# show ip route ospf

R4#show ip ospf database

R4#ping 10.33.4.1

On R1, R2, and R4, only one O E2 entry can now be seen in the routing table for the external networks. This represents the summary advertised in a Type-5 AS External LSA by R3. Notice that the OSPF database shows that R4 is receiving only one Type-5 AS External LSA from R3 (3.3.3.3).

R4 should still have connectivity to all external networks but its routing table is now much simpler.

Task 3: Configure OSPF Filtering

Activity

OSPF Filtering

Internal routing protocol filtering presents some special challenges with link-state routing protocols like OSPF. Link-state protocols do not advertise routes—instead, they advertise topology information. Also, SPF loop prevention relies on each router in the same area having an identical copy of the LSDB for that area. Filtering or changing LSA contents in transit could conceivably make the LSDBs differ on different routers, causing routing irregularities.

IOS supports four types of OSPF route filtering:

- ABR Type-3 LSA filtering using the filter-list command—A process of preventing an ABR from creating certain Type-3 summary LSAs.

- Using the area range not-advertise command—Another process to prevent an ABR from creating specific Type-3 summary LSAs.

- Filtering routes (not LSAs)—Using the distribute-list in command, a router can filter the routes that its SPF process is attempting to add to its routing table, without affecting the LSDB. This type of filtering can be applied to Type-3 LSAs and Type-5 LSAs.

- Using the summary-address not-advertise command—Like the area range not-advertise command but it is applied to the ASBR to prevent it from creating specific Type-5 AS External LSAs.

Filtering Type-3 LSAs with the Filter-List Command

ABRs do not forward Type-1 and Type-2 LSAs from one area into another, but instead create Type-3 LSAs for each subnet defined in the Type-1 and Type-2 LSAs. Type-3 LSAs do not contain detailed information about the topology of the originating area; instead, each Type-3 LSA represents a subnet, and a cost from the ABR to that subnet.

The OSPF ABR Type-3 LSA filtering feature allows an ABR to filter Type-3 LSAs at the point where the LSAs would normally be created. By filtering at the ABR, before the Type-3 LSA is injected into another area, the requirement for identical LSDBs inside the area can be met, while still filtering LSAs.

Note

All addressing and naming conventions used in these Learning Points will differ from the actual lab steps that you will perform.

To configure Type-3 LSA filtering, you use the area area-number filter-list prefix prefix-list-name in | out command under OSPF configuration mode. The referenced prefix-list is used to match the subnets and masks to be filtered. The area-number and the in | out option of the area filter-list command work together, as follows:

- When out is configured, IOS filters prefixes coming out of the configured area.

- When in is configured, IOS filters prefixes going into the configured area.

Step 1: On R1, configure an OSPF filter-list to prevent the 192.168.2.0/24 prefix from being advertised out of Area 1. Clear the OSPF process on R1 after applying the filter-list.

On R1, enter the following commands:

Using the ip prefix-list command, create a prefix list that denies 192.168.2.0/24 but permits all other prefixes. Under the OSPF routing process, assign the prefix list to Area 1 in the outbound direction. Applying a filter-list outbound of an area prevents all other areas that the ABR is connected to from receiving the prefix. Clearing the OSPF process allows the new policy to take effect immediately.

R1(config)# ip prefix-list FROM_R2 deny 192.168.2.0/24

R1(config)# ip prefix-list FROM_R2 permit 0.0.0.0/0 le 32

R1(config)# router ospf 1

R1(config-router)# area 1 filter-list prefix FROM_R2 out

R1(config-router)# end

R1# clear ip ospf process

Reset ALL OSPF processes? [no]: yes

Step 2: On R1, verify the OSPF configuration and routing table. On R3 and R4, examine the OSPF database and routing table.

On R1, R3, and R4, enter the following commands:

R1# show ip ospf | begin Area 1

R1#show ip route ospf

R3#show ip ospf database

R4#show ip ospf database

R4#show ip route 192.168.2.0

As the ABR, R1 is showing that the area filter has been applied outbound for Area 1. Notice that the routing table on R1 still has the 192.168.2.0/24 entry since it is considered an intra-area route for Area 1.

The OSPF database on R3 and R4 shows that R1 has stopped advertising the Type-3 LSA for 192.168.2.0/24 out of Area 1. The 192.168.2.0/24 route is also missing from their routing tables.

Step 3: On R1, change the filter list so that it is applied inbound on Area 0 instead. Clear the OSPF process on R1 after making the appropriate change.

On R1, enter the following commands:

R1(config)# router ospf 1

R1(config-router)# no area 1 filter-list prefix FROM_R2 out

R1(config-router)# area 0 filter-list prefix FROM_R2 in

R1# clear ip ospf process

Reset ALL OSPF processes? [no]: yes

R1#

After removing the filter list from Area 1, a new filter list is configured inbound for Area 0. Applying the filter list inbound for Area 0 will prevent the Type-3 LSA for prefix 192.168.2.0/24 from being advertises into area 0, but the prefix should still be advertised in area 2. Clearing the OSPF process allows the new policy to take effect immediately.

Step 4: On R1, verify the OSPF configuration and routing table. On R3 and R4, examine the OSPF database and routing table.

On R1, R3, and R4, enter the following commands:

R1# show ip ospf | begin Area

R1#show ip route ospf

R3#show ip ospf database

R3#show ip route ospf

R4# show ip route 192.168.2.0

The filter list has been applied to filter Type-3 LSAs when they are being advertised into Area 0 only. R1 still has the 192.168.2.0/24 prefix in its routing table as an intra-area route. You can see in R3’s OSPF database that the Type-3 LSA for the 192.168.2.0/24 prefix is being successfully advertised into area 2. R3 has also added the route to its routing table. R4 is no longer receiving a Type-3 LSA for 192.168.2.0/24. The route to that network has also been removed from its routing table.

Filtering Type-3 LSAs with the area range Command

The second method to filter OSPF routes is to filter Type-3 LSAs at an ABR using the area range command. The area range command performs route summarization at ABRs, telling a router to cease advertising smaller subnets in a particular address range, instead creating a single Type-3 LSA whose address and prefix encompass the smaller subnets. When the area range command includes the not-advertise keyword, not only are the smaller component subnets not advertised as Type-3 LSAs, but the summary route is also not advertised as a Type-3 LSA either. As a result, this command has the same effect as the area filter-list command with the out keyword, filtering the LSA from going out to any other areas.

- The second method to filter OSPF routes is to filter Type-3 LSAs at an ABR using the area range command.

- The area range command performs route summarization at ABRs, telling a router to cease advertising smaller subnets in a particular address range, instead creating a single Type-3 LSA whose address and prefix encompass the smaller subnets.

- When the area range command includes the not-advertise keyword, not only are the smaller component subnets not advertised as Type-3 LSAs, but the summary route is also not advertised as a Type-3 LSA either.

- As a result, this command has the same effect as the area filter-list command with the out keyword, filtering the LSA from going out to any other areas.

Step 5: On R1, use the area range command with the not-advertise option to filter the 192.168.3.0/24 prefix from being advertised out of area 2.

On R1, enter the following commands:

R1(config)# router ospf 1

R1(config-router)# area 2 range 192.168.3.0 255.255.255.0 not-advertise

The 192.168.3.0/24 prefix from area 2 is filtered at the ABR and not flooded into Area 0 or Area 1 as Type-3 LSAs. As you saw previously in the summarization task, the area range command is more commonly used to advertise a range of networks to other areas within an OSPF domain. In this case, using the not-advertise option, R1 will suppress the 192.168.3.0/24 prefix from being advertised out of area 2 as a Type-3 LSA.

Step 6: On R1, R2, and R4, use the various show commands to review the OSPF configuration, routes, and database information.

On R1, R2, and R4, enter the following commands:

R1# show ip ospf | begin Area 2

R1#show ip route ospf | include 192.168.3.0/24

R2#show ip ospf database

R2#show ip route 192.168.3.0

R4#show ip ospf database

R4#show ip route 192.168.3.0

The show ip ospf command on R1 is showing that the 192.168.3.0/24 will not be advertised out of area 2 but R1’s own routing table still has the intra-area prefix listed.

R2 and R4 are no longer receiving the 192.168.3.0/24 Type-3 LSA from R1 and the prefix has been removed from their routing tables.

Filtering OSPF Routes with the Distribute-List Command

For OSPF, the distribute-list in command filters what ends up in the IP routing table, and only on the router on which the distribute-list in command is configured. It does not remove routes from the link-state database of area routers. The process is straightforward, with the distribute-list command referencing either to an ACL, prefix list or route map.

The following rules govern the use of distribute lists for OSPF:

- The distribute list in the inbound direction applies to results of SPF—the routes to be installed into the router’s routing table.

- The distribute list in the outbound direction applies only to redistributed routes and only on an ASBR; it selects which redistributed routes shall be advertised. Redistribution is beyond the scope of this course.

- The inbound logic does not filter inbound LSAs; it instead filters the routes that SPF chooses to add to its own local routing table.

In the following steps you will use three different methods of filtering with the distribute-list command:

- Using an access list with the distribute-list command

- Using a prefix list with the distribute-list command

- Using a combination of route map, prefix list and access list with the distribute-list command.

Task 4: Using an Access List with the Distribute-List Command

Activity

Step 1: On R1, configure an ACL to prevent the 192.168.4.0/24 prefix from being added to its routing table. Apply the ACL to an inbound distribute list configured under the OSPF routing process.

On R1, enter the following commands:

R1(config)# access-list 1 deny 192.168.4.0 0.0.0.255

R1(config)# access-list 1 permit any

R1(config)# router ospf 1

R1(config-router)# distribute-list 1 in

The ACL will allow R1 to learn all routes except for the 192.168.4.0/24 prefix. The ACL is used as a distribute list and applied in the inbound direction to filter OSPF routes that are being added to its own routing table.

Step 2: On R1, examine the OSPF protocol parameters, the OSPF database, and the routing table.

On R1, enter the following commands:

R1# show ip protocols | section ospf

R1#show ip ospf database router 4.4.4.4

R1#show ip route 192.168.4.0

The show ip protocols command confirms that ACL 1 is applied as an incoming update filter.

Using the show ip ospf database router 4.4.4.4 command on R1 shows that the router is still receiving the 192.168.4.024 Type-1 LSA from R4 but as you can see, the prefix is not being added to R1’s routing table. The 4.4.4.4 address used with the show ip ospf database router command refers to the link state ID advertising the 192.168.4.0/24 network.

Step 3: On R2 and R3, examine the OSPF database and the routing table.

On R2 and R3, enter the following commands:

R2# show ip ospf database summary 192.168.4.0

R2#show ip route 192.168.4.0

R3#show ip ospf database summary 192.168.4.0

R3# show ip route 192.168.4.0

R2 and R3 are both receiving a Type-3 LSA for the 192.168.4.0/24 prefix and are adding the route to their routing table.

Task 5: Using a Prefix List with the Distribute-List Command

Activity

Step 1: On R1, modify the distribute list so that it uses a prefix list instead of an ACL to filter out the 192.168.4.0 route from its routing table. On R1, enter the following commands:

Start by creating the prefix list which will deny the 192.168.4.0/24 route but permit all others.

Remove the distribute list which currently references the ACL and configure a new distribute list that references the prefix list instead. On R1, enter the following commands:

R1(config)# router ospf 1

R1(config-router)# no distribute-list 1 in

R1(config-router)# distribute-list prefix R4-LOOP in

This new distribute list is also applied inbound to filter routes being added to R1’s routing table.

Step 2: On R1, examine once again the OSPF protocol parameters, the OSPF database and, the routing table.

On R1, enter the following commands:

R1# show ip protocols | section ospf

R1#show ip ospf database router 4.4.4.4

R1#show ip route 192.168.4.0

The show ip protocols command shows that a prefix list called R4-LOOP is now being used as the incoming update filter for OSPF.

As was the case previously with the ACL, the prefix list is also successfully filtering the 192.168.4.0/24 prefix from being added to the routing table, but the Type-1 LSA is still being received by R1.

Step 3: On R2 and R3, examine the OSPF database and the routing table.

On R2 and R3, enter the following commands:

R2# show ip ospf database summary 192.168.4.0

R2#show ip route 192.168.4.0

R3#show ip ospf database summary 192.168.4.0

R3# show ip route 192.168.4.0

R2 and R3 are still both receiving a Type-3 LSA for the 192.168.4.0/24 prefix and are adding the route to their routing table.

Task 6: Using a Combination of Route Map, Prefix List, and Access List with the Distribute-List Command

Activity

Step 1: On R4, configure a prefix list that matches the two WAN networks (172.16.12.0/30 and 172.16.13.0/30). Use a route map to filter out these routes from being added to R4’s routing table. Using an ACL, also ensure that the route map checks that R1 is the source of the Type-3 LSA to be filtered.

Start by defining the prefix list that will deny the two WAN prefixes but allow all other routes. On R4, enter the following commands:

R4(config)# ip prefix-list WAN-LINKS permit 172.16.12.0/30

R4(config)# ip prefix-list WAN-LINKS permit 172.16.13.0/30

Notice that the prefix list is using permit statements to match the WAN networks. You will be using the prefix list to match routes (permit) that will not be inserted into the routing table, but the actual filtering (deny) will happen with the route map.

Configure an ACL that matches R1’s router ID (1.1.1.1). R1 is the source of the Type-3 LSA that we are filtering. On R4, enter the following commands:

R4(config)# ip access-list standard R1-ID

R4(config-std-nacl)# permit 1.1.1.1

The advantage of using a route map for route filtering is that it gives you improved flexibility and granularity. You will combine the prefix list and the ACL to ensure that only the WAN links advertised in Type-3 LSAs by R1 will be filtered out of the routing table.

Construct a route-map that will match the WAN-LINKS prefix list and the R1-ID named ACL. On R4, enter the following commands:

R4(config)# route-map WAN-FILTER deny 10

R4(config-route-map)# match ip address prefix-list WAN-LINKS

R4(config-route-map)# match ip route-source R1-ID

R4(config-route-map)# route-map WAN-FILTER permit 20

R4(config-route-map)# router ospf 1

R4(config-router)# distribute-list route-map WAN-FILTER in

The route map contains two statements. Line 10 denies routes that match the WAN-LINKS prefix list AND the R1-ID access-list. Both matches must be true for the deny action to be applied. Line 20 permits all other routes, allowing R4 to add other Type-3 LSA routes to its routing table. The route map is applied inbound as a distribute list under the OSPF process.

Step 2: On R4, examine the OSPF protocol parameters, the OSPF database and the routing table.

On R4, enter the following commands:

R4# show ip protocols | begin ospf 1

R4#show ip ospf database

R4#show ip route ospf

The show ip protocols command confirms that the WAN-FILTER route map is applied as an incoming update filter for OSPF. The show ip protocols command confirms that the WAN-FILTER route map is applied as an incoming update filter for OSPF.

The LSDB still shows that R4 is receiving and storing the Type-3 LSAs for 172.16.12.0/30 and 172.16.13.0/30 from R1, but those routes have been filtered from the routing table thanks to the route map.

Filtering Type-5 LSAs with the Summary-Address Command

Recall that Type-5 AS External LSAs are originated by an ASBR (router redistributing external routes) and flooded through the whole OSPF autonomous system.

You cannot limit the way this LSA is generated except by controlling the routes redistributed into OSPF. When a Type-5 AS External LSA is being generated, it uses the RIB contents and honors the summary-address commands if configured.

It is then possible to filter Type-5 AS External LSAs on the ASBR in a similar way that was used to filter Type-3 LSAs on the ABR. Using the summary-address not-advertise command allows you to specify which external networks should be flooded across the OSPF domain as Type-5 AS External LSAs.

Recall that Type-5 AS External LSAs are originated by an ASBR (router redistributing external routes) and flooded through the whole OSPF autonomous system. It is then possible to filter Type-5 AS External LSAs on the ASBR in a similar way that was used to filter Type-3 LSAs on the ABR. Using the summary-address not-advertise command allows you to specify which external networks should be flooded across the OSPF domain as Type-5 AS External LSAs.

Step 3: On R3, define a new Loopback interface with IP address 10.33.33.1/24.

On R3, enter the following commands:

R3(config)# interface Loopback 1

R3(config-if)# ip address 10.33.33.1 255.255.255.0

To demonstrate this type of Type-5 AS External LSA filtering, define a new Loopback interface that will be advertised by R3 as another external network. The new network is not included in the ASBR summarization you applied earlier in Task 2.

Step 4: On R1, examine the OSPF database and the OSPF routing table for the 10.33.33.0/24 prefix.

On R1, enter the following commands:

R1# show ip ospf data external 10.33.33.0

R1#show ip route 10.33.33.0

R1 is receiving a Type-5 AS External LSA for the 10.33.33.0/24 prefix and has installed that route in its routing table as an O E2 entry.

Step 5: On R3, configure the summary-address command to prevent the 10.33.33.0/24 external prefix from being advertised into the OSPF domain.

On R3, enter the following commands:

R3(config-if)# router ospf 1

R3(config-router)# summary-address 10.33.33.0 255.255.255.0 not-advertise

R3 is configured with the summary-address not-advertise command for the 10.33.33.0/24 prefix. This prevents R3 from generating a Type-5 AS External LSA for that network.

Step 6: On R1, examine once again the OSPF database and the OSPF routing table for the 10.33.33.0/24 prefix.

On R1, enter the following commands:

R1# show ip ospf database external 10.33.33.0

R1# show ip route 10.33.33.0

Notice that the Type-5 AS External LSA is no longer present in the LSDB on R1, and the

10.33.33.0/24 prefix has been removed from its routing table.